SOP #05-05

Field Sanitation - Reservoir Maintenance

Created: 4/21/08

Updated by: Jonathan Dinsmore

Purpose:

To insure the integrity of the water supply coming from reservoirs.

Policy:

A sanitary survey of the reservoir and also the condition of the area around it shall be completed no less than once per season during a pre-planting inspection to insure the reservoir is properly maintained.

Program:

The following are concerns to identify and correct regarding a reservoir:

- Contamination of well which has supplied water to the reservoir

  - Drain reservoir and allow to dry

- Animal intrusion

  - Look for animal tracks around reservoir

  - Look for animal excrement around reservoir

  - Look for animal carcasses

- Contamination by influent

  - Look for any incoming water that is dirty and may be draining into the reservoir

- Surrounding vegetation

  - Look for trees that have limbs extending over the reservoir

  - Remove tall weeds that might attract animals, rodents or fowls

  - Remove as many weeds as practical from inside the reservoir

- Overflow pipe

  - Observe whether the pipe’s opening is clean and free of weeds and debris