SOP #05-06

Field Sanitation – Well-Water Maintenance

Created: 4/21/08

Updated by: Jonathan Dinsmore

Purpose:

The purpose of this procedure is to insure the integrity of the water supply coming from wells. A sanitary survey of the well and evaluation of the area around it shall be completed no less than once per season during a pre-planting inspection.

Policy:

Each well shall be inspected for proper operating conditions. This should be done no less than seasonally during the pre-planting inspection to insure the integrity of the well and security of the water to prevent contamination.

Program:

The following issues and recommendations shall be addressed and inspected when performing a sanitary survey on a well. All corrective actions shall be documented appropriately.

- Casing issues

  - Listen for water running down the well

  - Check to see if casing is loose

  - Make sure casing does not extend more than one foot out of the ground

- Well cap or seal

  - Well should be completely sealed; check for cracks

  - Look for holes, missing plugs, leaking water

- Well vent

  - Check the  cleanliness and integrity of the vent

  - Look for holes or tears

- Concrete well pads

  - Check for oil leaks

  - Check for corrosion

  - Check for excessive wear of the pump and shaft

- Cleanliness

  - Look for weeds, debris, trash

  - Look for evidence of small rodents

- Leaks and spills

  - Look for oil leaks from tanks, lines, etc.

  - Look for gasoline or diesel spills from supply tanks